Using these instructions and the standard components found in the ModelLibrary, a complete AW2000 system can be built.

Overall, there are 12 standard Cell Kits for the AW2000.  For the ModelLibrary only 6 of these kits are provided.  The difference in the kits is the wireway that feeds the cables to the Common Equipment Base as shown in the following.

The standard kits provided are:

- CELL_KIT_2M_SINGLE_MA1440	2 meter span with 1 MA1440 robots

- CELL_KIT_3M_DUAL_MA1440	3 meter span with 2 MA1440 robots

- CELL_KIT_3M_SINGLE_MA2010	3 meter span with 1 MA2010 robot

- CELL_KIT_4M_DUAL_MA2010	4 meter span with 2 MA2010 robots

- CELL_KIT_4M_SINGLE_MA3120	4 meter span with 1 MA3120 robot

- CELL_KIT_5M_DUAL_MA3120	5 meter span with 2 MA3120 robots

The AW2000 systems can also include one or two Stations, which can consist of 1 or 2  Standard postitioners or none at all.  The models for the positioners can be found in the ModelLibrary.

Models of other standard components such as torches, wirefeeders, welders, Controllers, etc. can also be found in the ModelLibrary

The example cell that will be built is an AW2200 using 2 MA3120 robots and 2 MHT1655 positioners with a 5 meter span.

- Create a new cell containing the required robot(s) and positioner(s).

- Open the ModelLibrary from the Home tab and change the Category to “AW2000 Cell Components”.

- Click, hold and drag the required Cell Kit to the center of the floor.

For this cell, the model “CELL_KIT_5M_DUAL_MA3120” was added.

Note: When adding models from the ModelLibrary, they can be added by dragging and dropping them on the desired parent, or they can be added by double-clicking on the item in the ModelLibrary.  When added by double-clicking, the model is positioned at the world frame position.  When dragged and dropped, the model is placed at the position of the cursor at the time the mouse button is released.

- Select the cell kit in the CADTree and verify its’ position is as shown.

Note: All components for the AW2000 must have their initial position set to zero as shown above, after being added to the cell.

The Cell Kits have dummy models in them for the purpose of locating the robots, positioners and torch stations.  If the model to be added is dropped on the appropriate dummy model, the component being added will be positioned correctly.

- In the Model Library “AW2000 Cell Components” category, select the FENCE _MA3120_CENTER _KIT and add it to the cell.

- If this model is dropped onto a surface of the CELL_KIT_5M_DUAL_MA3120, it will be added as a child model.

- The position in the CADTree must be set to zero as previously stated.

Note: All Fence Kit models except for ST1 Blocking were created for a 2 meter span and will need to be adjusted to the longer lengths.

- The model, FENCE_KIT-CENTER_MA3120-1, must have its’ position changed for the 5 meter span.  Set the position of this model as shown.

- Set the position of its’ child model, FENCE_KIT-CENTER_MA3120-2, as shown.

- Add the Station 1 fence kit for the MHT555/MHT1655.

- If this model is dropped onto a surface of the CELL_KIT_5M_DUAL_MA3120, it will be added as a child model.

- The position in the CADTree must be set to zero as previously stated.

- The model FENCE_MHT555-1655_ST1-1 must have its’ position changed for the 5 meter span.  Set the position of this model as shown.

- Set the position of its’ child model, FENCE_MHT555-1655_ST1-2, as shown.

- Add the Station 2 fence kit and set the model positions as in the previous 2 steps.

- Add the 5 meter Cable Support and set its’ position as shown.

- Change the ModelLibrary Category to “Torch Clean Stations”.  Drag and Drop the required torch cleaning station to the dummy model indicated in the following image.

- If the red dot as shown in the following image, is positioned on the dummy model, the model of the torch cleaning station will be added as a child model of that dummy, and will be positioned correctly.  Its’ orientation may need to be changed.

- Add the torch cleaning station for the second robot by dropping it in the dummy frame indicated.

Note: By adding all of the above components as child models of the Cell Kit they can quickly be hidden by collapsing the CADTree and hiding the parent model.

- Add the required robot risers by selecting the “Risers and BasePlates” Category in the ModelLibrary and dragging and dropping them on the appropriate dummy model.

Default risers for the systems are as follows:

- CELL_KIT_2M_SINGLE_MA1440 - 150697-3, 610mm

- CELL_KIT_3M_DUAL_MA1440 - 150697-3, 610mm

- CELL_KIT_3M_SINGLE_MA2010 - 150697-3, 610mm

- CELL_KIT_4M_DUAL_MA2010 - 150697-3, 610mm

- CELL_KIT_4M_SINGLE_MA3120 – 146026-3, 460mm

- CELL_KIT_5M_DUAL_MA3120 – 146026-3, 460mm

- Move the robots to the correct positions.  The following is one method that can be used.

  - Open the position box and use the settings as shown in the following.  Pick on the top-center of the riser and the robot will move to the correct X and Y position.

- For the MA3120, the initial Z value is 540 mm before picking the position.

- After picking on the top center of the riser, add this amount to the Z value.

- Set the Rz of Robot #1 to 180 if positioning the robot by this method.

- Display the names of the Positioner Station dummy frames.

- Hide (or delete) the dummy frames of the unused Positioner locations.

- Change the ModelLibrary Category to “Positioners”.  Drag and Drop the required positioner to the dummy model STATION1_555-1655_HS_POSITION.

Note: When only 1 positioner is to be used, it will be located at the Station 2 position.

- After dropping the positioner on the dummy frame, its’ position is set correctly.  The orientation will need to be set.

- Set the correct span for the tailstock of the positioner.

- The MHT555 and MHT1655 in the ModelLibrary were created with a 1 meter span.

- The MHT3155 in the ModelLibrary was created with a 1.5 meter span.

- Repeat the previous steps for Station2 positioner if used.

Your cell should now look like the following.

- Set up the control of the positioners using the instuctions in Adding YAI Standard Single Axis Positioners Using ModelLibrary.doc

# Note:The instuction are in the “Positioners” section of the MotoSim EG VRC Model Library found at https://www.motoman.com/motosimlib/default

- The “AW2000 Cell Components” Category of the ModelLibrary has a model of the Optional Inner Light Curtians.  Add this if desired. It is not required for the simulation.

- The “AW2000 Cell Components” Category of the ModelLibrary contains models for the Common Equipment Bases, which can be added if needed.

  - Add these as a child model of the Cell Kit or the world.

  - The models of these bases were positioned for the 2 meter systems.  They will need to be offset depending on the span of the system being used.

  - The bases contain dummy models for locating controllers, water circulators and welders.

  - Controllers, Water Circulators, Welding Power Sources and Robot Power disconnects with and without Comarc can be added.  These models can be found in other Catagories of the ModelLibrary.

- Add the Operator Stations from the ModelLibrary if desired.